# TUTORIAL: External Interrupt

LED Toggle with Push-Button

Name:  Duwon Yang					ID: 21900427

# I. Introduction

In this tutorial, we will learn how to use External Interrupt. We will create functions that capture the falling edge trigger by pushing a button using an external interrupt.

The objectives of this lab are to learn how to

Configure External input (EXTI) interrupt with NVIC

Create your own functions for configuration of interrupts

### Hardware

NUCLEO -F411RE

### Software

Keil uVision IDE, CMSIS, EC_HAL

### Documentation

STM32 Reference Manual

# II. Basics of External Interrupt (EXTI)

## A. Register List

List of external interrupt (EXTI) registers used in this tutorial [Reference Manual ch7, ch10.2]

| Type | Register Name | Description |
| --- | --- | --- |
| SYSCFG | SYSCFG_EXTICRx | External Interrupt Configuration, x=1 to 4 EXTICR1: for pin0~pin3 , EXTICR2: for pin4~pin7, etc |
| EXTI_ | EXTI_IMR | Interrupt Mask |
|  | EXTI_FTSR EXTI_RTSR | Falling/Rising Trigger Selection |

Schematic

## B. Register Setting

(Digital Input Setting)

- Enable GPIO peripheral clock                 	RCC->AHB1ENR

- Configure DigitalIn pin

(EXTI Setting)

- Enable SYSCFG peripheral clock. 		RCC->APB2ENR

- Connect the corresponding external line to GPIO 	SYSCFG->EXTICR

- Configure the trigger edge. 			EXTI->FTSR/RTSR

- Configure Interrupt mask 			EXTI->IMR

- Enable EXTI. 					EXTI->IMR

(NVIC Setting)

- Configure the priority of EXTI interrupt request. 	NVIC_SetPriority()

- Enable EXTI interrupt request. 			NVIC_EnableIRQ()

(EXTI Use)

- Create user codes in handler			EXTIx_IRQHandler()

- Clear pending bit after interrupt call

# III. Tutorial

## A. Register Configuration

1. Pin Initialization & Set LED and Push-button

LED: Port A Pin 5 / Output / Push-Pull / No Pull-Up & No Pull-Down

Push-button: Port C Pin 13 / Input / No Pull-Up & No Pull-Down

void setup(void){ RCC_HSI_init()l GPIO_init(GPIOA, 5, OUTPUT); GPIO_otype(GPIOA, 5, EC_PUSH_PULL); GPIO_pupd(GPIOA, 5, EC_NONE); GPIO_init(GPIOC, 13, INPUT); GPIO_pupd(GPIOC, 13, EC_NONE);

}

2. Enable Peripheral Clock: SYSCFGEN

- RCC_APB2ENR: Enable SYSCFG

RCC->APB2ENR |= 1<<14;

3. EXTI Initialization & Connect Push-button to EXTI line

- SYSCFG_EXTICR4: Connect PC_13(push-button) to EXTI13 line

SYSCFG->EXTICR[3] &=~ 15≪4   		 // clear bits [3:0]

SYSCFG->EXTICR[3] |=   2<<4  // set to 0010 for PC[13]

- EXTI_FTSR: Enable Falling Trigger

EXTI->FTSR |= 1<<13  // TR13=1

- EXTI_IMR: Interrupt NOT masked (Enable)

EXTR->IMR |= 1<<13 // MR13 = 1

## B. Programming

Procedure

- Create a new folder ‘EC/Tutorial/TU_EXTI/’

- Open the program ‘Keil uVision5’ and create a new project.

- Name the project as ‘TU_EXTI’.

- Create a new item called ‘TU_EXTI.c’ and use the given source code Click here to download

- This is an example code for turning LED on/off with the button input trigger.

- Fill in the empty spaces in the code.

- Run the program and check your result.

- Your tutorial report must be submitted to LMS

## Appendix

See here for MCU resources

- Pin Configuration of NUCLE-F401RE

- LED/Button Circuit Diagram